NEONATAL VENTILATION DECISION TREE — QUICK REFERENCE

Level II / II+ Nursery  |  32 Weeks to Term  |  Pediatric Emergency CDS Project  |  March 2026

VENTILATION ESCALATION PATHWAY

| STEP | MODALITY | SETTINGS | ESCALATE TO NEXT STEP IF: |
| --- | --- | --- | --- |
| STEP 0 | Room Air Observation | No intervention. Monitor SpO2, RR, WOB. | SpO2 < 90% at >10 min age OR mild tachypnea (RR >60) |
| STEP 1 | Low-Flow Nasal Cannula | 0.25-2 L/min. FiO2 titrate to SpO2 90-95%. | FiO2 > 0.30 OR Silverman ≥ 4 OR grunting/retracting |
| STEP 2 | CPAP | 5-7 cmH2O. Short binasal prongs. OG tube for decompression. FiO2 titrate. | FiO2 > 0.40 on CPAP 6-7 cmH2O OR PCO2 >60 + pH <7.25 OR apnea x3/hr OR Silverman ≥7 |
| STEP 3 | NIPPV or HFNC | NIPPV: PIP 15-20, PEEP 5-6, Rate 20-30. HFNC: 4-8 L/min. | FiO2 >0.40-0.50 OR PCO2 >60-65 + pH <7.20 OR refractory apnea OR exhaustion |
| STEP 4 | Intubation + Mech Vent | SIMV+VG: Vt 4-6 mL/kg, Rate 30-60, PEEP 5-6, FiO2 titrate. | Activate NICU transport. SBAR to receiving neonatologist. |

SURFACTANT DECISION BRANCH (From Step 2)

| WHEN TO GIVE | INSURE TECHNIQUE | DOSING |
| --- | --- | --- |
| FiO2 > 0.30-0.40 on CPAP + CXR consistent with RDS (ground glass + air bronchograms) + GA 32-36 wks | Intubate → Surfactant via ETT → Extubate to CPAP within 15-30 min (if adequate resp drive). LISA: thin catheter via laryngoscopy on CPAP (avoids intubation). | Poractant (Curosurf): 200 mg/kg initial (2.5 mL/kg). Beractant (Survanta): 100 mg/kg (4 mL/kg). ⚠ REDUCE PIP IMMEDIATELY post-dose (compliance improves within minutes). |

CPAP FAILURE CRITERIA — THE “RED FLAGS”

| FiO2 > 0.40 on CPAP 6-7 cmH2O | PCO2 > 60 + pH < 7.25 | Apnea > 3/hr or any needing PPV | Silverman ≥ 7 | Hemodynamic instability |
| --- | --- | --- | --- | --- |

VENTILATOR INITIAL SETTINGS BY PATHOLOGY

|  | Mode | Vt (VG) | PIP | PEEP | Rate | Ti | FiO2 / SpO2 Target |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RDS | SIMV+VG | 4-5 mL/kg | 18-22 | 5-6 | 40-60 | 0.30-0.35 | Start 0.30 / 90-95% |
| MAS | SIMV+VG | 5-6 mL/kg | 20-25 | 4-5 | 30-50 | 0.35-0.45 | Start 0.40-1.0 / 90-95% |
| PPHN | AC+VG | 5-6 mL/kg | 22-28 | 5-6 | 40-60 | 0.30-0.40 | 0.80-1.0 / 92-97% pre-ductal |
| TTN | CPAP preferred | N/A | N/A | 5-6 | N/A | N/A | Titrate / 92-97% |

ETT SIZE AND DEPTH

| Weight | GA | ETT (mm) | Depth at Lip (cm) | Suction Cath (F) |
| --- | --- | --- | --- | --- |
| 1.5-2.0 kg | 32-34 wk | 3.0 | 7.0-7.5 | 6 |
| 2.0-3.0 kg | 34-36 wk | 3.0-3.5 | 7.5-8.5 | 6-8 |
| 3.0-4.0 kg | 37-40 wk | 3.5 | 9.0-10.0 | 8 |
| >4.0 kg | >40 wk | 3.5-4.0 | 10.0-10.5 | 8-10 |

ETT depth (cm) = weight (kg) + 6  |  Confirm with CO2 detector + CXR (tip at T1-T2)

CAFFEINE CITRATE — LOAD EARLY

| Loading Dose | Maintenance | When to Give | Why It Matters |
| --- | --- | --- | --- |
| 20 mg/kg IV over 30 min | 5-10 mg/kg/day IV or PO q24h | At admission for any preterm with resp distress; 30 min before extubation | Reduces CPAP failure, reduces apnea, facilitates extubation. One of the safest neonatal drugs. |

BLOOD GAS TARGETS AND VENTILATOR ADJUSTMENTS

| Parameter | Target | If Too High | If Too Low |
| --- | --- | --- | --- |
| pH | 7.25-7.40 | ↑ pH (alkalosis): ↓ rate or Vt (respiratory). If metabolic: assess cause | ↓ pH (acidosis): ↑ rate or Vt (respiratory). Volume/vasopressors (metabolic) |
| PCO2 | 40-55 (RDS); 35-45 (PPHN) | Hypercarbia: ↑ rate first, then Vt | Hypocarbia (<30): ↓ rate/Vt. Causes cerebral vasoconstriction! |
| PaO2 | 50-80 mmHg | ↓ FiO2 first, then ↓ PEEP/MAP | ↑ FiO2 first. R/O pneumothorax, tube malposition |
| SpO2 | 90-95% (pre); 92-97% (term) | Wean FiO2 by 5% decrements | ↑ FiO2; assess for complications |

EXTUBATION READINESS CRITERIA

✓ FiO2 < 0.30-0.35 ✓ Rate weaned to 15-20 bpm ✓ PIP < 16-18 cmH2O (or low PIP with VG) ✓ PEEP 5 cmH2O ✓ Adequate spontaneous respiratory drive ✓ Caffeine loaded ≥ 30 min before extubation ✓ No significant A/B events ➜ EXTUBATE TO CPAP 5-6 cmH2O or NIPPV. NEVER extubate to bare nasal cannula.

WEANING PATHWAY

| Mechanical Ventilation | → | CPAP 5-7 cmH2O | → | HFNC 2-4 L/min or LFNC |
| --- | --- | --- | --- | --- |
| Wean FiO2, rate, PIP to extubation criteria |  | Wean FiO2 to <0.25; CPAP to 5; stable 12-24h |  | Wean to RA when SpO2 stable >92% on <0.25 FiO2 |

SILVERMAN-ANDERSEN RETRACTION SCORE

| Parameter | 0 = None | 1 = Moderate | 2 = Severe | Action |
| --- | --- | --- | --- | --- |
| Upper Chest | Synchronized | Lag on inspiration | Seesaw (paradoxical) |  |
| Lower Chest | None | Just visible | Marked |  |
| Xiphoid | None | Just visible | Marked |  |
| Nasal Flaring | None | Minimal | Marked | Score 0: No distress |
| Exp. Grunt | None | With stethoscope | Audible | 1-3: Mild (O2) 4-6: Mod (CPAP) 7-10: Severe (intubate) |

TRANSPORT CRITERIA

• Mechanical ventilation expected > 24 hours • FiO2 > 0.50 sustained • Suspected PPHN (pre/post-ductal gradient) • Need for iNO or HFOV • CPAP failure requiring intubation in infant < 32 weeks • Suspected surgical condition • Any condition beyond Level II/II+ scope

GREEN = Normal/Reassuring  |  YELLOW = Monitor/Recheck  |  RED = Intervene/Escalate  |  BLUE = Information/Protocol

References in full textbook. Educational use only. Verify all doses and settings against institutional protocols.